| FEATURES: 18 CR2032 Lithium Battery 210mAh Very Low Power Consumption Active Mode Sleep Mode Stop Mode Flickering Mode Seconds, Minutes, Hours Automatic Week Day, Day, Month, & Year 3 Alarms Settings, Sound & Lights (Optional) Buzzer Built In Auto Power OFF (After 15 Seconds) Manual Power OFF 138 Colored LED Lights Key Tone Operation (Optional) Hourly Chime (Optional) AM/PM Indicator Alarm Activated Display Green, Yellow, & Red Colors Supported SWITCHES: 2 Switch 1: Press to access Screens 1 to 8 Switch 2: Press to Shut OFF display Switch 1& 2: Press & hold simultaneously for Time Setting SCREENS: 8 Screen 1 = (1 press) Hour, Minute, & Seconds Screen 2 = (2 presses) Day of the week Screen 2 = Alarm 1, 2, 3, Hourly Chime, & Key Tone Screen 3 = (3 presses) Year, Month, Day of the Month Screen 4 = (4 presses) Alarm 1 Screen 5 = (5 presses) Alarm 2 Screen 6 = (6 presses) Alarm 3 Screen 7 = (Hourly Chime Setting.) Screen 8 = (Key Tone Setting) MODES: 4 Active Mode = Display ON (clock running) Sleep Mode = Display OFF (clock running) Stop Mode = Display OFF (clock stop running) Flickering Mode = Flashing lights (Hour, Minute & Second) CODES: 11 Red = Hours, Minutes & Seconds Red = 1st-5th seconds Yellow = 6th-10th Seconds Green = 11th-15th Seconds Then the seconds colour repeat cycle Hours 1-12 = Years: e.g. Hour 1 = Year 2001 etc. (image d) Hours 12-6 = Days of week e.g. Hour 12 = Sat, Hour 3 = Tues (image c) Minutes 5,10,15,20, etc, = Months e.g. Minute 10 = Feb (image d) Seconds 1-31 = Days of the Month e.g. Second 10 = The 10th (image d) Flashing light (Key Tone, Hourly Chime) = OFF Stable light (Key Tone, Hourly Chime) = ON AM = light OFF PM = light ON (Only when set) IMPORTANT NOTICE: Your watch will initially be in Stop Mode, drawing very little current from the battery. If you receive your watch in Stop Mode, you will have to set it to Active Mode to start using it. To do this please complete the followings: Wake up from Stop mode. Press & hold Switch 1 until you hear two beep ### Your Watch is ready to use! *After 15 Seconds (Active Mode) Displays will be deactivated “Display must be in “Active Mode” for all Settings” | 1/ TIME & DATE SETTINGS: Press & hold Switch 1 & Switch 2 simultaneously until Seconds light flashes Press & hold Switch 1 to bring the Seconds to 00 position Press Switch 2 once to set the desired Second Minute’s light will flash Press Switch 1 repeatedly to change the Minute Press Switch 2 once to set the desired Minute Hour’s light will flash Press Switch 1 repeatedly to change the Hour, AM or PM Press Switch 2 once to set the desired Hour Year’s Light will flash (Each Hour = 1 Year) Press Switch 1 repeatedly to change the Year Press Switch 2 once to set the Year Month’s light will flash (Fifth of each Minute) Press Switch 1 repeatedly to change the Month Press Switch 2 once to set the Month Days of the Month light will flash (Second 1 to second 31) Press Switch 1 repeatedly to change the Day Press Switch 2 to set the desired day Note: Day of Week will set it self Automatically (Hour 12 = Sunday) 2/ ALARM SETTINGS: Setting Alarm 1 Press & hold Switch 1 until Alarm 1 light flashes then release it Press Switch 2 once to activate the Minute Press Switch 1 repeatedly to set the desired Minute Press Switch 2 once to activate the Hour Press Switch 1 repeatedly to set the desired Hour, AM or PM Press Switch 2 to set Alarm 1 ON Setting Alarm 2 Press & hold Switch 1 until Alarm 1 light flashes then release it Press Switch 1 once to activate Alarm 2. Press Switch 2 once to activate the Minute. Press Switch 1 repeatedly to set the desired Minute Press Switch 2 once to activate the Hour. Press Switch 1 repeatedly to set the desired Hour, AM or PM Press Switch 2 to set Alarm 2 ON Setting Alarm 3 Press & hold Switch 1 until Alarm 1 light flashes then release it Press Switch 1 once to activate Alarm 2 Press Switch 1 once again to activate Alarm 3 Press Switch 2 to activate the Minute Press Switch 1 repeatedly to set the desired Minute Press Switch 2 to activate the Hour Press Switch 1 repeatedly to set the desired Hour, AM or PM Press Switch 2 to set Alarm 3 ON Checking Alarms ON/OFF: Alarms ON if in screen 2 (2 presses) Alarms lights are ON Alarms OFF if in screen 2 (2 presses) Alarms lights are OFF Checking Alarms Setting: Press Switch 1 4 times to see Alarm 1 Setting (screen 4) Press Switch 1 once again to see Alarm 2 Setting (screen 5) Press Switch 1 once again to see Alarm 3 Setting (screen 6) Press Switch 1 once again to leave the screen. Switching Alarms OFF: Press Switch 1 4 times to go to Alarm 1 (screen 4) Press Switch 2 once to switch Alarm 1 OFF Press Switch 1 once to go to Alarm 2 (screen 5) Press Switch 2 once to switch Alarm 2 OFF Press Switch 1 once to go to Alarm 3 (screen 6) Press Switch 2 once to switch Alarm 3 OFF Press Switch 1 once to leave the screen Note: When Alarms are OFF, Screens 4, 5, 6 are unavailable | 3/ HOURLY CHIME SETTING: Turn Hourly Chime ON/OFF: Press & hold Switch 1 until Alarm 1 flashes then release it Press Switch 1 once again to go to Alarm 2 Press Switch 1 once again to go to Alarm 3 Press Switch 1 once again to go to Hourly Chime (Screen 7) Press Switch 2 repeatedly to set Hourly Chime ON or OFF Press Switch 1 2 times to leave the screen Checking Hourly Chime: Press Switch 1 once to go to Time Reading Screen Press Switch 1 once again to see if Hourly Chime light is ON Press Switch 2 once to leave the screen 4/ KEY TONE SETTING: Turn Key Tone ON/OFF: Press & hold Switch 1 until Alarm 1 flashes Press Switch 1 once again to go to Alarm 2 Press Switch 1 once again to go to Alarm 3 Press Switch 1 once again to go to Hourly Chime (Screen 7) Press Switch 1 once again to go to Key Tone (Screen 8) Press Switch 2 repeatedly to set Key Tone ON or OFF Press Switch 1 once to leave the screen Checking Key Tone: Press Switch 1 once to go to Time Reading Screen. Press Switch 1 once again to see if Key Tone light is ON Press Switch 2 once to leave the screen ENTER Stop mode Press & hold Switch 2 until you hear two beep Your Watch is in stop mode. ENTER FLICKERING MODE IN SLEEP MODE : Press & hold Switch 2 Flickering Mode is ON Repeat the same procedure to turn the Flickering Mode OFF BATTERY REPLACEMENT: 1/ Battery should last approx. 3/4 years if Display is activated up to 25/30 times a day 2/ Replace Battery when lights start to dim PICTURE 1 Battery Type: CR2032 / 3V |  |
| --- | --- | --- | --- |